# SWMS 102 – WORK AT HEIGHTS: ERECTING AND WORKING FROM SCAFFOLDING

| NOTE: This industry standard SWMS template has been made accessible by Breffni Ceilings & Partitions Pty Ltd, to be used as a guide only. The sub-contractor/user is entirely responsible for its contents at the point of submission. Therefore, please ensure that every section is reviewed and personalised in full. |  |  |  |
| --- | --- | --- | --- |
| - PROJECT DETAILS |  |  |  |
| Project Name: |  | Project Address: |  |
| Client / Principal Contractor: |  |  |  |
| Description of Work (Scope): |  |  |  |
| High Risk Construction Work: |  |  |  |
| Date Work is to Commence: |  | Approx. Duration of Work: |  |
| Person Responsible for Monitoring Activity: |  | Name: |  |
| Date Created: |  | Issue: | A |

| - CONSULTATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| THIS SWMS HAS BEEN DEVELOPED IN CONSULTATION WITH THE FOLLOWING WORKERS: |  |  |  | SWMS PROVIDED TO CLIENT / PRINCIPAL CONTRACTOR |  |  |  |
| Issue: | Name: | Position: | Date: | Issue: | Name: | Position: | Date: |
| A |  |  |  | A |  |  |  |

| - APPROVAL |  |  |  |
| --- | --- | --- | --- |
|  | APPROVED BY | REVIEWED PRIOR TO WORK COMMENCING (Principal Contractor Representative) | REVIEWED ON SITE PRIOR TO COMMENCING WORK (Site Supervisor) |
| NAME |  |  |  |
| SIGNATURE |  |  |  |
| DATE |  |  |  |

| - PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uniform | Footwear | High Vis’ | Safety Helmet | Protective Eyewear | Face Shield | Fall Arrest | Gloves | Hearing | Dust Mask | Respirator | Other | PPE. Notes (e.g. P2 Dust Mask, etc.) |
|  |  |  |  |  |  |  |  |  |  |  |  | Other mandatory site PPE as advised by Client or PC. |

| - PERMITS, TICKETS OR LICENCES REQUIRED TO PERFORM WORK |  |
| --- | --- |
| Role / Task | Permit, Ticket or Licence |
| Workers | Company Induction, General Construction Induction |
| Assemble or dissemble scaffolding -prefabricated, bracket, tank and formwork scaffolds | Scaffolding licence - Class: SB – Basic Scaffolder Licence |
| Assemble or dissemble scaffolding except hung or suspended scaffolds | Scaffolding licence - Class: SI – Intermediate Scaffolder Licence |
| Assemble or dissemble hung or suspended scaffolds | Scaffolding licence - Class: SA – Advanced Scaffolder Licence |

| - PLANT AND EQUIPMENT |  |  |  |
| --- | --- | --- | --- |
| Plant and Equipment Description | Maintenance / Checks / Tests | Plant and Equipment Description | Maintenance / Checks / Tests |
| Adjustable wrench, Podger hammer, scaffold spanner, spirit level, rule and tape measures stored in a scaffold belt. | Pre start inspection of equipment will be carried out. Podger hammers with a loose head or a hairline crack at the juncture between the head and the shaft will be replaced. Scaffold spanners with a worn rivet or a sloppy head will be repaired or replaced. | Mandatory site PPE: steel capped boots, hard hat, hi-visibility vest as required. | Checked before use by worker to ensure they are in good condition. |
|  |  | Fall protection | Visually inspected before use, tested as per manufacturer’s instructions. |

| - CHEMICALS |  |  |  |
| --- | --- | --- | --- |
| Chemical Name | Hazardous? | SDS Attached | Controls |
|  | Yes No | Yes No | As per Safety Data Sheet |
|  | Yes No | Yes No |  |
|  | Yes No | Yes No |  |
|  | Yes No | Yes No |  |

| - LEGISLATION AUSTRALIAN STANDARDS AND CODES OF PRACTICE TO BE COMPLIED WITH |  |
| --- | --- |
| QUEENSLAND | NATIONAL |
| Work Health and Safety Act 2011 | AS/NZS 1576.1:2010 Scaffolding – General Requirements |
| Work Health and Safety Regulation 2011 | AS/NZS 1576.2:2016 Scaffolding - Couplers and accessories |
| Electrical Safety Act 2002 (Qld) | AS/NZS 1576.4:2013 Scaffolding - Suspended scaffolding |
| Electrical Safety Regulation 2013 (Qld) | AS/NZS 1576.5:1995 Scaffolding - Prefabricated splitheads and trestles |
| How to Manage Work Health and Safety Risks Code of Practice 2011 | AS/NZS 1577:2013 Scaffold Decking Components |
| Scaffolding Code of Practice 2009 | AS/NZS 4576:1995 Guidelines for Scaffolding |
| Managing Noise and Preventing Hearing Loss at Work Code of Practice 2011 |  |
| Managing the Risk of Falls at Workplaces Code of Practice 2011 |  |
| Managing Risks of Plant in the Workplace Code of Practice 2013 |  |
| Hazardous Manual Tasks Code of Practice 2011 |  |
| Electrical Safety Code of Practice 2010 – Working Near Overhead and Underground Electric Lines |  |
| WHS Consultation, Co-operation and Co-ordination Code of Practice 2011 |  |

| - WORK RISKS AND CONTROLS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item No. | Job Procedure in Steps | Possible Health, Safety and Environmental Hazards/ Risks | Risk Score Before | Health, Safety and Environmental Controls | Risk Score After | Person(s) Responsible |
|  | Pre-start | Workers not understanding risk controls | E 80 | Pre-start meeting with workers to discuss requirements of this SWMS. Feedback will be requested from workers. All workers to sign off accordingly. | M 52 | Site Supervisor |
|  | Selection of equipment | Inappropriate equipment being utilised | E 80 | Make sure scaffolding is: - Designed in accordance with AS/NZS 4576; - Maintained in accordance with AS/NZS 4576. Inspection testing and maintenance; - Marked with safe working load (SWL); - Labelled with advice on safe means of access and if not insulated - ‘DO NOT USE WHERE ELECTRICAL HAZARDS EXIST’; - Appropriate for the required task considering height, safe working load and ground conditions. | L40 | Worker Site Supervisor Contractor |
|  | Siting | Contact with overhead power lines | E 88 | Identify and avoid overhead powerlines. Do not erect scaffolding within 4 m of powerlines. Enforce exclusion zones. Follow SWMS Work near Power Lines. | M 52 | Worker Site Supervisor Contractor |
|  |  | Scaffold collapse- vehicular traffic | E 80 | Reroute traffic away from the location of the scaffold. Utilise safety observer or licensed traffic controller as necessary. Install guards or fenders if practical. | L 40 | Worker Site Supervisor Contractor |
|  | Preparing site | Injury to a member of the public | E 72 | Establish an exclusion zone using barricading or other suitable means around the work area. Areas around scaffolds being erected or dismantled will be barricaded off as required. Store equipment out of walkways. | L 40 | Worker Site Supervisor Contractor |
|  | Working on or near a road | Injury to pedestrians Collision involving road vehicles | E 80 | Implement Traffic controls. Refer to SWMS Work near Roads. Obtain necessary approvals from local authorities if required. | L 40 | Worker Site Supervisor Contractor |
|  | Erecting scaffolding where there is a risk of falling less than 4 metres | Equipment or component failure/fault | E 72 | Inspect equipment prior to use. Check and test in accordance with the manufacturer’s pre-operational checklist. If fault is found that affects the safe use of the scaffold: - Do not use Scaffold; - Tag as ‘Out of Service’; and - Repair or remove from the work area (if possible). | L 40 | Worker Site Supervisor Contractor |
|  |  | Scaffold collapse - erected incorrectly | E 80 | Erect according to verified design or the supplier’s documented information. Erect scaffold in accordance with suppliers’ instructions, AS/NZS 1576.1, AS 1576.2, AS 1576.4 and AS/NZS 1576.5. | M 52 | Worker Site Supervisor Contractor |
|  |  | Scaffold collapse - unsuitable footings (ground unable to hold weight) Damage to ground surface | E 80 | Foundation of scaffold adequate to carry and distribute the loads imposed. Protect surface with sole plates if necessary (centrally support base plates). | M 52 | Worker Site Supervisor Contractor |
|  |  | Scaffold collapse - stability of scaffold | E 72 | Ensure the stability of for instance by: - Tying off to supporting structure. - Guying to a supporting structure. - Attaching counter weights near the base. - Adding backup bays to increase the base dimension. - Wind speed should be monitored while work activity is underway, along with the forecast wind condition and any weather alerts issued to provide advance warning of high winds. | L 40 | Worker Site Supervisor Contractor |
|  |  | Scaffold collapse - rigidity of scaffolding | E 72 | Utilise diagonal bracing systems. | L 40 | Worker Site Supervisor Contractor |
|  | Erecting prefabricated platform units or scaffolding planks | Trip/falls | H 60 | Make sure planks or decking: - Are uniform thickness to prevent uplift or displacement; - Are placed to avoid significant gaps and tripping hazards; and - Are butted not lapped except at returns, curved faces or unusual profiles. | L 40 | Worker Site Supervisor Contractor |
|  |  | Safe access and travelling within scaffolding | E 72 | Make sure working platforms are wide enough to accommodate materials and plant and allow clear and unobstructed access along its entire length. Platforms on scaffolding must be at least 450mm wide for the full length of the platform or cover the entire width of the scaffold frame, whichever the greater distance. | L 40 | Worker Site Supervisor Contractor |
|  |  | Fall from heights | E 88 | Make sure guardrail is in place between 900mm and 1100mm above the platform and the midrail approx. halfway between toeboard and guardrail. Make sure guardrails and midrails are supported at intervals not exceeding 3m parallel with the platform and not further than 100mm outside the platform edge. Guardrails, where erected to prevent falls from heights, must be erected to comply with AS 1657. | M 52 | Worker Site Supervisor Contractor |
|  |  | Safe access | E 72 | Any access ladders must extend 1m beyond the stepping off point and ladder must be secured to prevent movement. | L 40 | Worker Site Supervisor |
|  |  | Falling objects | E 72 | Do not walk under scaffolding without an overhead catch-platform or gantry in place or containment sheeting in place. Tethers must be used to tie-off tools, equipment and materials. | L 40 | Worker Site Supervisor |
|  | Working against the face of a structure/ object where there is no guardrail between scaffold and structure/ object | Fall from heights | E 72 | Ensure that the gap between the platform edge and the structure is less than 225 mm. | L 40 | Worker Site Supervisor |
|  | Erecting scaffolding where there is a risk of a person or thing falling more than 4 metres | Fall from heights (more than 4 metres) | E 88 | Scaffolds will be designed by a qualified person and a scaffold plan is to be developed and followed. Submit work plans prior to work commencing. The scaffold plan shall be strictly complied with. The scaffold plan shall form part of the project documentation. | M 52 | Worker Site Supervisor |
|  |  | Incorrectly erected scaffold | E 80 | Licensed scaffolders will erect scaffolding as prescribed by relevant safety legislation. | M 52 | Worker Site Supervisor |
|  |  | Fall from heights | E 88 | Make sure where a person can fall at least 2 metres in erecting the scaffolding, the following is implemented: - A fall arrest harness system is in place Follow SWMS Work at Heights: Using Fall Arrest Systems; - OR - A fall control measure is in place such as access by way of a ladder within the scaffold with decking planks shall be cleated or otherwise secured to prevent displacement; - OR - The following are installed immediately after enough components of the scaffolding have been erected to support it: - Edge protection across the space between the uprights forming the outer frame of the scaffolding at the level the scaffolding has reached; and - A means of access to the level the scaffolding has reached; and - Each time a new level of scaffolding is erected, a platform is installed below the level at a distance of not more than 2 metres. | M 52 | Worker Site Supervisor |
|  | Commissioning scaffolding | Scaffolding used before it is completed or safe | E 80 | Non-completed or scaffolds not in accordance with the scaffold plan will be signed as “NO ACCESS.” Provide or obtain a certificate of completion if required. | M 52 | Worker Site Supervisor |
|  | Using fixed scaffolding where there is a risk of falling less than 4 metres | Overloading of scaffolding | E 72 | Do not exceed Safe Working Loads as prescribed by the manufacturer. Make sure dead and live loads are calculated. | L 40 | Worker Site Supervisor |
|  | Using fixed scaffolding where there is a risk of falling more than 4 metres | Structural failure | E 80 | Scaffolds, mobile or fixed, are to be inspected at start of day or start of work. Defective or incomplete scaffolds shall not be used. | M 52 | Worker Site Supervisor |
|  |  | Rescue injured or stranded worker | E 80 | Make sure personnel and equipment are readily available to perform rescue in case of emergency. Do not work alone. Make sure an effective communication system is in place at all times. | L 40 | Worker Site Supervisor |
|  | Using mobile scaffolding | Unexpected movement | E 80 | Make sure castors on mobile scaffolding are locked when scaffolding is in use. Make sure the scaffolding is only moved from ground level and only when no one is on the scaffold. Only use mobile scaffolding on a hard level surface without sudden floor level changes. | M 52 | Worker Site Supervisor |
|  |  | Scaffold tipping over | E 80 | Make sure the height does not exceed 3 times the width of the smallest base dimension. Make sure ladder access is provided within the tower and not on the outside. Do not climb up or down the scaffold - use access provided. Make sure mobile scaffold is not located closer than one metre to any penetration, stepdown or slab edge. Climbing on incomplete scaffolds during erection stage, without adequate fall-protection, is prohibited. | M 52 | Worker Site Supervisor |
|  | Carrying equipment up scaffolding | Manual handling | E 72 | Use fibre rope to raise or lower heavy materials. Use tool belts where possible. | L 40 | Worker Site Supervisor |
|  | Monitoring | Failure of scaffold components | E 80 | Scaffolding will be inspected: - Before first use and within every 30 days; - After alteration or repair; and - After any event likely to affect the stability of the scaffold (for example, following strong winds or storms); - Inspections are recorded on the scaffold Inspection Record below. | L 40 | Worker Site Supervisor |
|  | Site Emergency | Poor or delayed response in the case of emergency | E 80 | Site specific emergency procedures will be followed at all times. | L 40 | Worker Site Supervisor |

| - RISK ASSESSMENT TOOL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RISK MATRIX |  |  |  |  |  |  |  |  | HIERARCHY OF CONTROLS |
| LIKELIHOOD |  |  | CONSEQUENCE |  |  |  |  |  |  |
|  |  |  | 1 |  | 2 | 3 | 4 | 5 |  |
|  |  |  | Insignificant |  | Minor | Moderate | Major | Catastrophic |  |
| A. | Almost certain |  | M 52 |  | H 64 | E 76 | E 88 | E 100 |  |
| B. | Likely |  | M 44 |  | H 56 | H 68 | E 80 | E 92 |  |
| C. | Possible |  | L 36 |  | M 48 | H 60 | E 72 | E 84 |  |
| D. | Unlikely |  | L 28 |  | L 40 | M 52 | H 64 | E 76 |  |
| E. | Rare |  | L 20 |  | L 32 | M 44 | H 56 | H 68 |  |
| SCORE INDICATOR |  |  |  |  |  |  |  |  |  |
| SCORE |  | LEVEL OF DANGER |  | REQUIRED ACTION |  |  |  |  |  |
| 72 – 100 |  | EXTREME |  | NO WORK SHOULD COMMENCE AT THIS LEVEL! This is an unacceptable level of risk. A review of controls must be performed to reduce risk as low as reasonably practicable. If risk cannot be further reduced, a Permit System must be applied. The permit must be available at all times work is being performed, signed by both the workers, a senior management representative and the Client (if applicable) to ensure that the risk is reduced as far as practicable. |  |  |  |  |  |
| 56 – 68 |  | HIGH |  | WORK CAN COMMENCE AT THIS LEVEL IF THE CONTROL MEASURES ARE APPROVED BY THE PROJECT MANAGER OR DELEGATE The Project Manager or delegate will review existing controls to ensure that the risk is reduced to as low as is reasonably practicable. |  |  |  |  |  |
| 44 – 52 |  | MODERATE |  | WORK CAN COMMENCE WITH CONTROL MEASURES IN PLACE. Management must monitor and review the controls to ensure controls are adequately used, do not present further hazards / risks and aim to reduce the risk further if practicable. |  |  |  |  |  |
| 20 - 40 |  | LOW |  | REVIEW IF CHANGES OCCUR. Record and review if any equipment, people, materials, work processes, or procedures change. |  |  |  |  |  |

| - MONITORING AND REVIEW OF CONTROL MEASURES |  |
| --- | --- |
| Pre-Start | Pre-start meetings are held to review control measures and ensure workers are aware of controls. |
| Ongoing Monitoring | Site Supervisor will continually monitor works against the controls stated in this safe work method statement. The Health and Safety Checklist is used to document formal inspections of the works and work area. |
| Annual | This safe work method statement is reviewed annually or as required. |
| NON-CONFORMANCE | Should any aspect of this SWMS not be complied with, work affected by the non-compliance must cease. The Site Supervisor will either enforce the controls as stated by the SWMS or, where appropriate, review the SWMS in consultation with workers and update with more suitable controls provided the revised controls still ensure that the hazards / risks are controlled as far as reasonably practicable. |

| - SWMS CONSULTATION AND SIGN-OFF RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I confirm, by signing below, that I have read and understand the requirements of this SWMS, that I agree that the SWMS is acceptable for the proposed work, and I agree to abide by the requirements. If the work conditions change, or additional hazards are identified, these changes shall be discussed, the SWMS modified and signed again. |  |  |  |  |  |
| ISSUE | NAME (Please Print) | SIGNATURE | ISSUE | NAME (Please Print) | SIGNATURE |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |