# SWMS 103 – WORK AT HEIGHTS: USING ELEVATING WORK PLATFORMS

| NOTE: This industry standard SWMS template has been made accessible by Breffni Ceilings & Partitions Pty Ltd, to be used as a guide only. The sub-contractor/user is entirely responsible for its contents at the point of submission. Therefore, please ensure that every section is reviewed and personalised in full. |  |  |  |
| --- | --- | --- | --- |
| - PROJECT DETAILS |  |  |  |
| Project Name: |  | Project Address: |  |
| Client / Principal Contractor: |  |  |  |
| Description of Work (Scope): |  |  |  |
| High Risk Construction Work: |  |  |  |
| Date Work is to Commence: |  | Approx. Duration of Work: |  |
| Person Responsible for Monitoring Activity: |  | Name: |  |
| Date Created: |  | Issue: | A |

| - CONSULTATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| THIS SWMS HAS BEEN DEVELOPED IN CONSULTATION WITH THE FOLLOWING WORKERS: |  |  |  | SWMS PROVIDED TO CLIENT / PRINCIPAL CONTRACTOR |  |  |  |
| Issue: | Name: | Position: | Date: | Issue: | Name: | Position: | Date: |
| A |  |  |  | A |  |  |  |

| - APPROVAL |  |  |  |
| --- | --- | --- | --- |
|  | APPROVED BY | REVIEWED PRIOR TO WORK COMMENCING (Principal Contractor Representative) | REVIEWED ON SITE PRIOR TO COMMENCING WORK (Site Supervisor) |
| NAME |  |  |  |
| SIGNATURE |  |  |  |
| DATE |  |  |  |

| - PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uniform | Footwear | High Vis’ | Safety Helmet | Protective Eyewear | Face Shield | Fall Arrest | Gloves | Hearing | Dust Mask | Respirator | Other | PPE. Notes (e.g. P2 Dust Mask, etc.) |
|  |  |  |  |  |  |  |  |  |  |  |  | Other mandatory site PPE as advised by Client or PC. |

| - PERMITS, TICKETS OR LICENCES REQUIRED TO PERFORM WORK |  |
| --- | --- |
| Role / Task | Training, Permit, Ticket or Licence |
| Workers | Company Induction, General Construction Induction |
| Operating EWP | EWP Competency Certificate OR Crane and Hoist operation - Class: WP |
| Controlling Traffic | Traffic Control Permit / Authority |
| 6 and 12 monthly Inspection of Travel restraint systems | Height Safety Equipment Inspector |

| - PLANT AND EQUIPMENT |  |  |  |
| --- | --- | --- | --- |
| Plant and Equipment Description | Maintenance / Checks / Tests | Plant and Equipment Description | Maintenance / Checks / Tests |
| Elevating Work Platform | Prestart check as per logbook | Fall arrest equipment | Visually inspected before use. Tested as per manufacturer’s instructions. |

| - CHEMICALS |  |  |  |
| --- | --- | --- | --- |
| Chemical Name | Hazardous? | SDS Attached | Controls |
|  | Yes No | Yes No | As per Safety Data Sheet |
|  | Yes No | Yes No |  |

| - LEGISLATION AUSTRALIAN STANDARDS AND CODES OF PRACTICE TO BE COMPLIED WITH |  |
| --- | --- |
| QUEENSLAND |  |
| Work Health and Safety Act 2011 | Managing Risks of Plant in the Workplace Code of Practice 2013 |
| Work Health and Safety Regulation 2011 | Traffic Management for Construction or Maintenance Work 2008 |
| Electrical Safety Act 2002 (Qld) | WHS Consultation, Cooperation and Coordination Code of Practice 2011 |
| Electrical Safety Regulation 2013 (Qld) | NATIONAL |
| Electrical Safety Code of Practice 2010 – Working Near Overhead and Underground Electric Lines | AS/NZS 1891.1:2007/Amdt 2:2008 Industrial Fall-Arrest Systems and Devices - Harnesses and ancillary equipment |
| Managing Noise and Preventing Hearing Loss at Work Code of Practice 2011 | AS/NZS 1891.3:1997 Industrial Fall-Arrest Systems and Devices - Fall Arrest Devices |
| Hazardous Manual Tasks Code of Practice 2011 | AS/NZS 1891.4:2009 Industrial Fall-Arrest Systems and Devices - Selection, Use and Maintenance |
| Managing the Risk of Falls at Workplaces Code of Practice 2011 | AS 2550.1-2011 Cranes, hoists and winches – Safe use – General Requirements |
| How to Manage Work Health and Safety Risks Code of Practice 2011 | AS 2550.10-2006/Amdt 1-2009 Cranes, hoists and winches - Safe use - Mobile elevating work platforms |
| Managing Risks of Hazardous Chemicals in the Workplace Code of Practice 2013 | AS 1418.10:2011 Cranes, hoists and winches – Mobile elevating work platforms |
|  | AS 1742 Set Manual of uniform control traffic devices set |

| - WORK RISKS AND CONTROLS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item No. | Job Procedure in Steps | Possible Health, Safety and Environmental Hazards/ Risks | Risk Score Before | Health, Safety and Environmental Controls | Risk Score After | Person(s) Responsible |
|  | Pre-start | Workers not understanding risk controls | E 80 | Pre-start meeting with workers to discuss requirements of this SWMS. Feedback will be requested from workers. All workers to sign off accordingly. | M 52 | Site Supervisor |
|  | Selection | Inappropriate equipment being utilised | E 88 | Make sure EWP is: - Designed in accordance with AS 1418.10 - Cranes, hoists and winches - Mobile elevating work platforms; - Maintained in accordance with AS 2550.10 Cranes, hoists and winches - Safe use - Mobile elevating work platforms; - Marked with plant design number and safe working load (SWL); - Appropriate for the required task considering height, reach, safe working load, ground conditions and terrain or restricted working space. | L 40 | Site Supervisor |
|  | Siting | Rollover | E 80 | Ensure plant is set up on a level, stable surface. Ensure the slope of the surface does not exceed the manufacturer’s recommendations. Assess likely wind conditions. Do not use if wind conditions are unsuitable. Set stabilizers or outriggers (if fitted). | M 52 | Worker Site Supervisor |
|  |  | Fall into excavations or down embankments Affect structural integrity of excavations or embankments | E 88 | Avoid setting up near excavations or embankments. | M 52 | Worker Site Supervisor |
|  |  | Blocking access ways or emergency exits | E 72 | Ensure that plant does not block access ways or emergency exits. | L 28 | Worker Site Supervisor |
|  | Pre-operational inspection/tasks | Mechanical failure/fault | E 72 | Check and test in accordance with the manufacturer’s pre-operational checklist. Make sure safety devices and interlocks are operational. Record checks in the logbook kept on the EWP. If fault is found that affects the safe operation of the EWP: - Do not use EWP; - Remove keys; - Tag as ‘Out of Service’; and - Remove from the work area (if possible). | L 40 | Worker Site Supervisor |
|  | Operation - Loading an EWP | Overloading of EWP | E 80 | Make sure EWP has SWL marked on the working platform. Make sure safe working load is not exceeded. Include personnel, tools and material being loaded in weight calculation. | M 52 | Worker Site Supervisor |
|  | Operation - General | Mechanical failure | E 80 | Do not place equipment or material near or on controls or hinged points of boom. | L 40 | Worker |
|  |  | Unexpected movement | E 88 | Only bucket controls are to be used while personnel are in the bucket. Except in emergency or for maintenance purposes. | M 52 | Worker Site Supervisor |
|  |  | Fall from height | E 88 | Keep your body within the confines of the working platform at all times. | M 52 | Worker |
|  |  | Noise - hearing damage and/ or loss | E 80 | Wear appropriate hearing protection i.e. ear muffs or plugs. | M 52 | Worker |
|  |  | Poor or delayed response in case of emergency | E 80 | Make sure at least one other person trained in the operation of the EWP is readily available to operate in case of emergency. Do not work alone. Make sure an effective communication system is in place at all times. | L 40 | Worker Site Supervisor |
|  | Operation - Scissor type EWP | Competency level of operator | E 88 | Only operate if you are competent to do so i.e. hold a certificate of competence or are training under the direct supervision of a person who holds appropriate qualifications. | M 52 | Worker Site Supervisor |
|  |  | Fall from heights | E 80 | Do not sit or stand on edge of bucket. | M 52 | Worker |
|  | Operation - Boom type EWP | Competency level of operator | E 88 | Only operate if you are competent to do so i.e. hold a certificate of competence or WP class licence if the boom length exceeds 11 metres or are training under the direct supervision of a person who holds appropriate qualifications. | M 52 | Worker Site Supervisor |
|  |  | Falls from heights | E 88 | Do not sit or stand on edge of bucket. Wear a full harness and lanyard system compliant to AS/NZS 1891.1 and AS/NZS 1891.4. Attach harness to approved anchor point. Inspect equipment before use. Do not use faulty or damaged equipment. | M 52 | Worker |
|  | Travelling with platform lowered | Impact with person or object | E 88 | Follow SWMS Work involving Movement of Mobile Powered Plant Make sure basket is in line with the direction of travel. Travel at manufacturer’s recommended speed. | M 52 | Worker Site Supervisor |
|  | Travelling with platform elevated | Impact with person or object | E 88 | Follow SWMS Work involving Movement of Mobile Powered Plant Travel at manufacturer’s designated creep speed (if this is not known then not more than 1 km/h). Make sure the operator has a clear view of the base structure. Do not travel with the platform elevated when: - Ground surface is unsuitable i.e. undulations, obstructions, and pot holes; - Persons are in the work area; and/or - Gradient is unsuitable. | M 52 | Worker Site Supervisor |
|  | Working on slopes/ gradients | Rollover | E 88 | Only work on surfaces within the manufacturer’s recommendations. Face bucket uphill. | M 52 | Worker |
|  | Transporting an EWP | EWP fall off or out of other vehicle | E 88 | Engage parking brakes or otherwise secure the EWP. Secure loose items to prevent movement. Securely stow the outrigger beams and footplates (if fitted). Secure the platform in the stowed position. Engage the slew lock mechanism. Securely attach to the vehicle in accordance with manufacturer’s recommendations. | L 40 | Worker Site Supervisor |
|  | Using an EWP as a means of access/ egress | Fall from heights | E 88 | Do not enter or leave the platform when elevated (except in an emergency) unless: - A risk assessment has been performed and documented; - EWP access to work area is considered the safest alternative; - The landing area is structurally sound; - Landing area is clear; - The gap between platform and landing area is not more than 100 mm or more than 2 m from the edge of the structure; - The platform is secured (e.g. tied) to a suitable point on the landing; - The safety harness is properly worn; - The safety harness is attached to a suitable anchorage on the structure; - The base controls are tagged to indicate the EWP is in use and to caution against interference. | M 52 | Worker Site Supervisor |
|  | Work over other personnel | Falling objects | E 80 | Tethers must be used to tie-off tools, equipment and materials. Establish an exclusion zone using barricading or other suitable means below the work area. No persons are permitted under the bucket. | L 40 | Worker Site Supervisor |
|  | Working on or near a road | Injury to pedestrians Collision involving road vehicles | E 88 | Implement traffic controls. Refer to SWMS Work Near Roads. Obtain necessary approvals from local authorities if required. | M 52 | Worker Site Supervisor |
|  | Working in a public place | Injury to pedestrians Collision involving road vehicles | E 80 | Use warning devices i.e. barricading and/or safety spotter and appropriate signage. Obtain necessary approvals from local authorities if required. Ensure other persons permitted in the exclusion area are wearing high visibility vests. | M 52 | Worker Supervisor |
|  | Work near power lines | Contact with power lines | E 88 | Identify and avoid overhead power lines. Enforce exclusion zones. | M 52 | Worker Site Supervisor |
|  | Whilst accessing the height | Emergency requiring retrieval from the EWP | E 72 | Workers will never work alone and will monitor each other whilst accessing the height or working at height. Prior to commencing work the operator will check that emergency lowering procedures are in functional. In the event that a worker needs to be lowered from the EWP the normal emergency lowering method will be utilised. | M 52 | Worker Site Supervisor |
|  |  | Failure of upper control functions while elevated | E 72 | Where the normal control function fails, the operator will use the auxiliary controls from the platform to lower the EWP safely to the ground. | M 52 | Worker Site Supervisor |
|  |  | Failure of the operator to be able to operate the EWP functions while elevated due to the following reasons: - Operator incapacity - Auxiliary functions fail to operate from the upper control station | E 72 | Where the operator is incapable of lowering the EWP using the upper controls, an appointed person familiarised in the use of the lower ‘ground’ controls will lower the platform safely to the ground using the lower ground controls. | M 52 | Worker Site Supervisor |
|  |  | Failure of lower ground controls | E 72 | Where the lower ground controls fail to allow the EWP to be lowered safely to the ground, the appointed person will use the auxiliary ground controls to lower the EWP safely to the ground. | M 52 | Worker Site Supervisor |
|  |  | Failure of all normal and auxiliary lowering functions | E 72 | Where all functions fail to work, emergency services will be notified. | M 52 | Worker Site Supervisor |
|  | Whilst working at height | Emergency requiring the retrieval of the worker from the work at height | E 72 | Worker will never work alone and will monitor each other whilst working at height. The EWP will be operated from the ground to utilise the EWP to reach the worker from the elevated position. | M 52 | Worker Site Supervisor |
|  | Site Emergency | Poor or delayed response in the case of emergency | E 80 | Site specific emergency procedures will be followed at all times. | L 40 | Worker Site Supervisor |

| - RISK ASSESSMENT TOOL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RISK MATRIX |  |  |  |  |  |  |  |  | HIERARCHY OF CONTROLS |
| LIKELIHOOD |  |  | CONSEQUENCE |  |  |  |  |  |  |
|  |  |  | 1 |  | 2 | 3 | 4 | 5 |  |
|  |  |  | Insignificant |  | Minor | Moderate | Major | Catastrophic |  |
| A. | Almost certain |  | M 52 |  | H 64 | E 76 | E 88 | E 100 |  |
| B. | Likely |  | M 44 |  | H 56 | H 68 | E 80 | E 92 |  |
| C. | Possible |  | L 36 |  | M 48 | H 60 | E 72 | E 84 |  |
| D. | Unlikely |  | L 28 |  | L 40 | M 52 | H 64 | E 76 |  |
| E. | Rare |  | L 20 |  | L 32 | M 44 | H 56 | H 68 |  |
| SCORE INDICATOR |  |  |  |  |  |  |  |  |  |
| SCORE |  | LEVEL OF DANGER |  | REQUIRED ACTION |  |  |  |  |  |
| 72 – 100 |  | EXTREME |  | NO WORK SHOULD COMMENCE AT THIS LEVEL! This is an unacceptable level of risk. A review of controls must be performed to reduce risk as low as reasonably practicable. If risk cannot be further reduced, a Permit System must be applied. The permit must be available at all times work is being performed, signed by both the workers, a senior management representative and the Client (if applicable) to ensure that the risk is reduced as far as practicable. |  |  |  |  |  |
| 56 – 68 |  | HIGH |  | WORK CAN COMMENCE AT THIS LEVEL IF THE CONTROL MEASURES ARE APPROVED BY THE PROJECT MANAGER OR DELEGATE The Project Manager or delegate will review existing controls to ensure that the risk is reduced to as low as is reasonably practicable. |  |  |  |  |  |
| 44 – 52 |  | MODERATE |  | WORK CAN COMMENCE WITH CONTROL MEASURES IN PLACE. Management must monitor and review the controls to ensure controls are adequately used, do not present further hazards / risks and aim to reduce the risk further if practicable. |  |  |  |  |  |
| 20 - 40 |  | LOW |  | REVIEW IF CHANGES OCCUR. Record and review if any equipment, people, materials, work processes, or procedures change. |  |  |  |  |  |

| - MONITORING AND REVIEW OF CONTROL MEASURES |  |
| --- | --- |
| Pre-Start | Pre-start meetings are held to review control measures and ensure workers are aware of controls. |
| Ongoing Monitoring | Site Supervisor will continually monitor works against the controls stated in this safe work method statement. The Health and Safety Checklist is used to document formal inspections of the works and work area. |
| Annual | This safe work method statement is reviewed annually or as required. |
| NON-CONFORMANCE | Should any aspect of this SWMS not be complied with, work affected by the non-compliance must cease. The Site Supervisor will either enforce the controls as stated by the SWMS or, where appropriate, review the SWMS in consultation with workers and update with more suitable controls provided the revised controls still ensure that the hazards / risks are controlled as far as reasonably practicable. |

| - SWMS CONSULTATION AND SIGN-OFF RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I confirm, by signing below, that I have read and understand the requirements of this SWMS, that I agree that the SWMS is acceptable for the proposed work, and I agree to abide by the requirements. If the work conditions change, or additional hazards are identified, these changes shall be discussed, the SWMS modified and signed again. |  |  |  |  |  |
| ISSUE | NAME (Please Print) | SIGNATURE | ISSUE | NAME (Please Print) | SIGNATURE |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |