# SWMS 109 – WORK NEAR POWER LINES

| NOTE: This industry standard SWMS template has been made accessible by Breffni Ceilings & Partitions Pty Ltd, to be used as a guide only. The sub-contractor/user is entirely responsible for its contents at the point of submission. Therefore, please ensure that every section is reviewed and personalised in full. |  |  |  |
| --- | --- | --- | --- |
| - PROJECT DETAILS |  |  |  |
| Project Name: |  | Project Address: |  |
| Client / Principal Contractor: |  |  |  |
| Description of Work (Scope): |  |  |  |
| High Risk Construction Work: | Work near an exposed energised electrical installation. |  |  |
| Date Work is to Commence: |  | Approx. Duration of Work: |  |
| Person Responsible for Monitoring Activity: |  | Name: |  |
| Date Created: |  | Issue: | A |

| - CONSULTATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| THIS SWMS HAS BEEN DEVELOPED IN CONSULTATION WITH THE FOLLOWING WORKERS: |  |  |  | SWMS PROVIDED TO CLIENT / PRINCIPAL CONTRACTOR |  |  |  |
| Issue: | Name: | Position: | Date: | Issue: | Name: | Position: | Date: |
| A |  |  |  | A |  |  |  |

| - APPROVAL |  |  |  |
| --- | --- | --- | --- |
|  | APPROVED BY | REVIEWED PRIOR TO WORK COMMENCING (Principal Contractor Representative) | REVIEWED ON SITE PRIOR TO COMMENCING WORK (Site Supervisor) |
| NAME |  |  |  |
| SIGNATURE |  |  |  |
| DATE |  |  |  |

| - PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uniform | Footwear | High Vis’ | Safety Helmet | Protective Eyewear | Face Shield | Fall Arrest | Gloves | Hearing | Dust Mask | Respirator | Other | PPE. Notes (e.g. P2 Dust Mask, etc.) |
|  |  |  |  |  |  |  |  |  |  |  |  | Other mandatory site PPE as advised by Client or PC. |

| - PERMITS, TICKETS OR LICENCES REQUIRED TO PERFORM WORK |  |
| --- | --- |
| Role / Task | Permit, Ticket or Licence |
| Workers | Company Induction, General Construction Induction |

| - PLANT AND EQUIPMENT |  |  |  |
| --- | --- | --- | --- |
| Plant and Equipment Description | Maintenance / Checks / Tests | Plant and Equipment Description | Maintenance / Checks / Tests |

| - CHEMICALS |  |  |  |
| --- | --- | --- | --- |
| Chemical Name | Hazardous? | SDS Attached | Controls |
|  | Yes No | Yes No | As per Safety Data Sheet |
|  | Yes No | Yes No |  |

| - LEGISLATION AUSTRALIAN STANDARDS AND CODES OF PRACTICE TO BE COMPLIED WITH |  |
| --- | --- |
| QUEENSLAND | NATIONAL |
| Work Health and Safety Act 2011 | AS 1418.1-2002/Amdt 1-2004 Cranes, hoists and winches—General requirements |
| Work Health and Safety Regulation 2011 | AS 1418.5-2013 Cranes, hoists and winches—Mobile cranes |
| Electrical Safety Act 2002 (Qld) | AS 1418.10:2011/Amdt 1:2017 Cranes, hoists and winches – Mobile elevating work platforms |
| Electrical Safety Regulation 2013 (Qld) | AS 2550.1-2011 Cranes, hoists and winches – Safe use – General Requirements |
| How to Manage Work Health and Safety Risks Code of Practice 2011 | AS 2550.5-2016 Cranes, hoists and winches – Safe use – Mobile cranes |
| Electrical Safety Code of Practice 2013 – Managing Electrical Risks in the Workplace | AS 2550.10-2006/Amdt 1-2009 Cranes, hoists and winches - Safe use - Mobile elevating work platforms |
| Electrical Safety Code of Practice 2010 – Works | AS 2550.15-1994 Cranes – Safe use – Concrete placing equipment |
| Electrical Safety Code of Practice 2010 – Working Near Overhead and Underground Electric Lines | AS/NZS 1576.1:2010 Scaffolding – General Requirements |
| Managing Noise and Preventing Hearing Loss at Work Code of Practice 2011 | AS/NZS 1576.5:1995 Scaffolding - Prefabricated splitheads and trestles |
| Hazardous Manual Tasks Code of Practice 2011 |  |
| Mobile Cranes Code of Practice 2006 |  |
| Managing Risks of Plant in the Workplace Code of Practice 2013 |  |
| WHS Consultation, Cooperation and Coordination Code of Practice 2011 |  |

| - WORK RISKS AND CONTROLS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item No. | Job Procedure in Steps | Possible Health, Safety and Environmental Hazards/ Risks | Risk Score Before | Health, Safety and Environmental Controls | Risk Score After | Person(s) Responsible |
|  | Pre-start | Workers not understanding risk controls | E 80 | Pre-start meeting with workers to discuss requirements of this SWMS. Feedback will be requested from workers. All workers to sign off accordingly. | M 52 | Site Supervisor |
|  | Working under the direction of a Client/Principal Contractor. | Contact with power lines – electrocution | E 92 | When working near sources of high sources of electricity, consult with the Client/Principal Contractor to ensure all requirements of the Electrical Safety Act are considered i.e. voltage and the recommended safe working distance. Only qualified and competent electrical workers are to undertake electrical tasks. | M 52 | Contractor Site Supervisor |
|  | Working near power lines. | Electrical shock as a result of equipment or persons making contact with a live overhead line. Electrical shock when electricity from a line arcs between the line and the equipment being used. Damage to overhead lines. | E 92 | Breffni Ceilings & Partitions Pty Ltd will comply with the Electrical Safety Act 2002 and Regulations 2013. No electrical work will be undertaken unless the persons performing the work are deemed as competent as prescribed under the Electrical Safety Act. Breffni Ceilings & Partitions Pty Ltd will not allow: - A person to come into direct contact with an electrical part unless that person is an authorised person or instructed person for the electrical part and the electrical part is low voltage overhead insulated electric line; - Any operating plant or vehicle to come into direct contact with an electrical part or within the exclusion zone unless the operating plant is being operated by an authorised person or instructed person and a safety observer is being used and the electrical part is low voltage overhead insulated electric line; - A person coming within an exclusion zone for the person for an electrical part. | M 52 | Site Supervisor |
|  | Identify voltages of electrical lines, | Electric shock Damage to overhead lines | E 92 | Voltages are detailed on power poles or can be obtained from the electrical provider. The maximum voltage is | M 52 | Worker Site Supervisor |
|  | Identify exclusion zones. | Electric shock Damage to overhead lines | E 92 | See EXCLUSION ZONES IN MILLIMETRES table attached. The exclusion zone for works is | M 52 | Worker Site Supervisor |
|  | Identify if consultation is required. | Not taking appropriate safety precautions. | E 92 | For work near High Voltage lines consultation allows for determining voltages of lines and communication of safety advice. If there is the likelihood of the work being performed may involve a person operating plant or a vehicle coming into direct contact with an overhead electric line or coming within the exclusion zone for the electric line and consult with the person in control of the electric lines. Written notice will be provided detailing the intention to perform the work. Electricity provider staff will determine the hazard potential and will advise on what procedure is to be adopted before work commences. Will not allow work to begin until safety advice is obtained by the person in control of the overhead electric line. For work near Low Voltage lines consultation allows for determining the voltage of the line and inform the owner of the line that you are carrying out work near the line. | M 52 | Worker Site Supervisor |
|  | Identify if a safety observer is required. | Persons or plant entering the exclusion zones. | E 92 | A safety observer will be appointed if cranes or plant are operated where any part of the crane of plant, any person, hand tool or other equipment or loads could enter the exclusion zone of the live electrical lines during operation. The safety observer is used to prevent encroachment into the exclusion zone. Proper communication needs to be put in place between the safety observer and the person controlling the crane or plant – i.e. two-way radio, hand signals or whistles. | M 52 | Worker Site Supervisor |
|  | Performing work. | Contact with or damage to electrical lines. | E 92 | Only trained and competent workers will be allowed to perform work. Exclusion zones will be enforced. | M 52 | Site Supervisor |
|  | Working within exclusion zones. | Contact with or damage to electrical lines. | E 92 | Working within exclusion zones will only be considered as a last option. If work within an exclusion zone cannot be avoided the requirement set out in the Electrical Safety Regulation 2013 for live electrical work (section 12) will be complied with. | M 52 | Worker Site Supervisor |
|  | Low voltage | Electrocution Serious bodily injury | E 92 | Without consultation, the exclusion zone is 3 metres. With consultation, the exclusion zone is as specified in the Exclusion Zones in Millimetres table. Breffni Ceilings & Partitions Pty Ltd will comply with the Queensland Electrical Safety Act 2002 and Regulations 2013. No electrical work will be undertaken unless the person is deemed as competent as prescribed under the Electrical Safety Act. | M 52 | Worker Site Supervisor |
|  | Underground powerlines | Electrocution Serious bodily injury | E 92 | Client/Principal Contractors are to advise the location of underground services. Before commencing any excavation in areas where underground lines are installed, or suspected to be installed, contact must be made with the appropriate service provider in order to obtain an accurate location of the lines in question. Once a location has been provided by the service provider, and prior to commencing major excavation with plant, the depth at which the power line is located must be established by careful excavation with plant and/or hand digging. Breffni Ceilings & Partitions Pty Ltd will comply with the Queensland Electrical Safety Act 2002 and Regulations 2013. No electrical work will be undertaken unless the persons performing the work are deemed competent as prescribed under the Electrical Safety Act. Australian Lawn Concepts will appoint competent persons to undertake excavation work. | M 52 | Worker Site Supervisor |
|  | Site Emergency | Poor or delayed response in the case of emergency | E 80 | Site specific emergency procedures will be followed at all times. | L 40 | Worker Site Supervisor |

| - RISK ASSESSMENT TOOL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RISK MATRIX |  |  |  |  |  |  |  |  | HIERARCHY OF CONTROLS |
| LIKELIHOOD |  |  | CONSEQUENCE |  |  |  |  |  |  |
|  |  |  | 1 |  | 2 | 3 | 4 | 5 |  |
|  |  |  | Insignificant |  | Minor | Moderate | Major | Catastrophic |  |
| A. | Almost certain |  | M 52 |  | H 64 | E 76 | E 88 | E 100 |  |
| B. | Likely |  | M 44 |  | H 56 | H 68 | E 80 | E 92 |  |
| C. | Possible |  | L 36 |  | M 48 | H 60 | E 72 | E 84 |  |
| D. | Unlikely |  | L 28 |  | L 40 | M 52 | H 64 | E 76 |  |
| E. | Rare |  | L 20 |  | L 32 | M 44 | H 56 | H 68 |  |
| SCORE INDICATOR |  |  |  |  |  |  |  |  |  |
| SCORE |  | LEVEL OF DANGER |  | REQUIRED ACTION |  |  |  |  |  |
| 72 – 100 |  | EXTREME |  | NO WORK SHOULD COMMENCE AT THIS LEVEL! This is an unacceptable level of risk. A review of controls must be performed to reduce risk as low as reasonably practicable. If risk cannot be further reduced, a Permit System must be applied. The permit must be available at all times work is being performed, signed by both the workers, a senior management representative and the Client (if applicable) to ensure that the risk is reduced as far as practicable. |  |  |  |  |  |
| 56 – 68 |  | HIGH |  | WORK CAN COMMENCE AT THIS LEVEL IF THE CONTROL MEASURES ARE APPROVED BY THE PROJECT MANAGER OR DELEGATE The Project Manager or delegate will review existing controls to ensure that the risk is reduced to as low as is reasonably practicable. |  |  |  |  |  |
| 44 – 52 |  | MODERATE |  | WORK CAN COMMENCE WITH CONTROL MEASURES IN PLACE. Management must monitor and review the controls to ensure controls are adequately used, do not present further hazards / risks and aim to reduce the risk further if practicable. |  |  |  |  |  |
| 20 - 40 |  | LOW |  | REVIEW IF CHANGES OCCUR. Record and review if any equipment, people, materials, work processes, or procedures change. |  |  |  |  |  |

| - MONITORING AND REVIEW OF CONTROL MEASURES |  |
| --- | --- |
| Pre-Start | Pre-start meetings are held to review control measures and ensure workers are aware of controls. |
| Ongoing Monitoring | Site Supervisor will continually monitor works against the controls stated in this safe work method statement. The Health and Safety Checklist is used to document formal inspections of the works and work area. |
| Annual | This safe work method statement is reviewed annually or as required. |
| NON-CONFORMANCE | Should any aspect of this SWMS not be complied with, work affected by the non-compliance must cease. The Site Supervisor will either enforce the controls as stated by the SWMS or, where appropriate, review the SWMS in consultation with workers and update with more suitable controls provided the revised controls still ensure that the hazards / risks are controlled as far as reasonably practicable. |

| - SWMS CONSULTATION AND SIGN-OFF RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I confirm, by signing below, that I have read and understand the requirements of this SWMS, that I agree that the SWMS is acceptable for the proposed work, and I agree to abide by the requirements. If the work conditions change, or additional hazards are identified, these changes shall be discussed, the SWMS modified and signed again. |  |  |  |  |  |
| ISSUE | NAME (Please Print) | SIGNATURE | ISSUE | NAME (Please Print) | SIGNATURE |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |

## Exclusion Zones in Millimetres Table

| Nominal phase to phase voltage (kV) (exposed unless otherwise stated) | Untrained Persons |  |  | Instructed Persons (IP) & Authorised Persons (AP) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Untrained Persons | Operation of operating plant | Operation of vehicle | IP & AP | Operation of operating plant, with safety observe or another safe system | Operation of vehicle |
| LOW VOLTAGE (LV) WITH CONSULTATION | 1000 | 3000 | 600 | No exclusion zone prescribed | 1000 | 600 |
| LOW VOLTAGE WITHOUT CONSULTATION | 3000 |  |  |  |  |  |
| LV & UPTO 33 WITH CONSULTATION | 2000 |  | 900 | 700 | 1200 | 700 |
| LV & UPTO 33 WITHOUT CONSULTATION | 3000 |  |  |  |  |  |
| 33 & UPTO 50 |  |  | 2100 | 750 1000 1200 | 1300 | 750 |
| 50 & UPTO 66 |  |  |  |  | 1400 | 1000 |
| 66 & UPTO 110 |  |  |  |  | 1800 |  |
| 110 & UPTO 132 |  |  |  |  |  | 1200 |
| 132 & UPTO 220 | 4500 | 6000 | 2900 | 1800 2300 | 2400 | 1800 |
| 220 & UPTO 275 | 5000 |  |  |  | 3000 | 2300 |
| 275 & UPTO 330 | 6000 |  | 3400 | 3000 | 3700 | 3000 |
| 330 & UPTO 400 |  | 8000 | 4400 | 3300 3900 | 4000 | 3300 |
| 400 & UPTO 500 |  |  |  |  | 4600 | 3900 |
| NOMINAL POLE TO EARTH DC VOLTAGE (KV) |  |  |  |  |  |  |
| +/- 25 | 3000 | 3000 | 900 | 700 | 1200 | 700 |
| +/- 85 | 3000 |  | 2100 | 1000 | 1800 | 1000 |
| +/- 150 | 3000 |  |  | 1200 |  | 1200 |
| +/- 270 | 4500 | 6000 | 2900 | 1800 | 2400 | 1800 |
| +/- 350 | 5000 |  |  | 2500 | 3200 | 2500 |
| +/- 400 | 6000 |  | 3400 | 2900 | 3600 | 2900 |
| Reference: ELECTRICAL SAFETY CODE OF PRACTICE 2010 - WORK NEAR OVERHEAD AND UNDERGROUND ELECTRIC LINES |  |  |  |  |  |  |