# SWMS 111 – WORK ON OR NEAR ASBESTOS

| NOTE: This industry standard SWMS template has been made accessible by Breffni Ceilings & Partitions Pty Ltd, to be used as a guide only. The sub-contractor/user is entirely responsible for its contents at the point of submission. Therefore, please ensure that every section is reviewed and personalised in full. |  |  |  |
| --- | --- | --- | --- |
| - PROJECT DETAILS |  |  |  |
| Project Name: |  | Project Address: |  |
| Client / Principal Contractor: |  |  |  |
| Description of Work (Scope): |  |  |  |
| High Risk Construction Work: | Involves, or is likely to involve, the disturbance of asbestos |  |  |
| Date Work is to Commence: |  | Approx. Duration of Work: |  |
| Person Responsible for Monitoring Activity: |  | Name: |  |
| Date Created: |  | Issue: | A |

| - CONSULTATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| THIS SWMS HAS BEEN DEVELOPED IN CONSULTATION WITH THE FOLLOWING WORKERS: |  |  |  | SWMS PROVIDED TO CLIENT / PRINCIPAL CONTRACTOR |  |  |  |
| Issue: | Name: | Position: | Date: | Issue: | Name: | Position: | Date: |
| A |  |  |  | A |  |  |  |

| - APPROVAL |  |  |  |
| --- | --- | --- | --- |
|  | APPROVED BY | REVIEWED PRIOR TO WORK COMMENCING (Principal Contractor Representative) | REVIEWED ON SITE PRIOR TO COMMENCING WORK (Site Supervisor) |
| NAME |  |  |  |
| SIGNATURE |  |  |  |
| DATE |  |  |  |

| - PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uniform | Footwear | High Vis’ | Safety Helmet | Protective Eyewear | Face Shield | Fall Arrest | Gloves | Hearing | Dust Mask | Respirator | Other | PPE. Notes (e.g. P2 Dust Mask, etc.) |
|  |  |  |  |  |  |  |  |  |  |  |  | Other mandatory site PPE as advised by Client or PC. P1 or better particulate respirator |

| - PERMITS, TICKETS OR LICENCES REQUIRED TO PERFORM WORK |  |
| --- | --- |
| Role / Task | Training, Permit, Ticket or Licence |
| Workers | Company Induction, General Construction Induction VET Course for Asbestos Removal |
| Supervisors | VET Supervise Asbestos Removal Course |
| Removal of any amount of friable Asbestos or removal of asbestos contaminated dust (ACD) or debris except where the ACD is associated with the removal of non-friable asbestos or is not associated with the removal of friable or non-friable asbestos and is only a minor contamination | ‘A’ Class Licence |
| Removal of 10m2 or more of non-friable Asbestos or asbestos containing material or removal of ACD associated with the removal of more than 10m2 of non-friable asbestos or asbestos containing material | ‘B’ Class Licence |
| Carry out air monitoring, clearance inspections and issuing of clearance certificates. | ‘A’ Class Asbestos Assessor: Must hold a VET course for Asbestos Assessors. |

| - PLANT AND EQUIPMENT |  |  |  |
| --- | --- | --- | --- |
| Plant and Equipment Description | Maintenance / Checks / Tests | Plant and Equipment Description | Maintenance / Checks / Tests |

| - CHEMICALS |  |  |  |
| --- | --- | --- | --- |
| Chemical Name | Hazardous? | SDS Attached | Controls |
|  | Yes No | Yes No | As per Safety Data Sheet |
|  | Yes No | Yes No |  |
|  | Yes No | Yes No |  |

| - LEGISLATION AUSTRALIAN STANDARDS AND CODES OF PRACTICE TO BE COMPLIED WITH |  |
| --- | --- |
| QUEENSLAND | NATIONAL |
| Work Health and Safety Act 2011 | AS/NZS 1716:2012 Respiratory protective devices |
| Work Health and Safety Regulation 2011 | Globally Harmonised System of Classification and Labelling of Chemicals |
| How to Manage Work Health and Safety Risks Code of Practice 2011 |  |
| Hazardous Manual Tasks Code of Practice 2011 |  |
| How to Manage and Control Asbestos in the Workplace Code of Practice 2011 |  |
| How to Safely Remove Asbestos Code of Practice 2011 |  |
| Managing Risks of Hazardous Chemicals in the Workplace Code of Practice 2013 |  |
| WHS Consultation, Co-operation and Co-ordination Code of Practice 2011 |  |

| - WORK RISKS AND CONTROLS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item No. | Job Procedure in Steps | Possible Health, Safety and Environmental Hazards/ Risks | Risk Score Before | Health, Safety and Environmental Controls | Risk Score After | Person(s) Responsible |
|  | Pre-start | Workers not understanding risk controls | E 80 | Pre-start meeting with workers to discuss requirements of this SWMS. Feedback will be requested from workers. All workers to sign off accordingly. | M 52 | Site Supervisor |
|  | Identifying Asbestos | Inhalation of Asbestos fibres Exposure of workers and public to Asbestos | E 88 | Owners and principal contractors are required to establish and maintain an Asbestos register than identifies and locates the locations of all Asbestos on sites built before 2004. To assess the site register and identify any areas where Asbestos may be encountered. If any work is to be conducted around Asbestos, management is to be notified immediately. | M 52 | Site Supervisor |
|  | Working around Asbestos | Cross contamination | E 80 | Do not eat, drink or smoke in the work area. Before any food or drink is consumed or smoking takes place, contaminated clothing must be removed and hands and face are to be thoroughly washed. | M 52 | Worker |
|  | Isolating the work area | Members of the public or other trades entering the work area | E 80 | Erect barriers, lock doors and where necessary erect warning signs. The positioning of safety barriers in relation to the work site will depend on the result of a risk assessment and consideration of the working environment and possibility of interference or intrusion by others. | M 52 | Worker Site Supervisor |
|  | Taking a sample of a suspected Asbestos containing material | Contact with Asbestos containing material/fibres | E 88 | Only samples of material which is damaged (i.e. does not require intact materials to be disturbed) will be taken by Breffni Ceilings & Partitions Pty Ltd workers. All other sampling will be performed by a qualified professional. Isolate the work area and wear appropriate PPE. Place a plastic sheet on the area below. Use a clean and sealable container to place the sample in. Label container and state when and where the sample was taken. Dispose of plastic sheet, PPE, Asbestos material and other waste according to the local council procedures. | M 52 | Worker Site Supervisor Contractor |
|  | Leaving Asbestos in place | Contact with Asbestos containing material | E 80 | Must be recorded in the building’s Asbestos Register. Sealed and or isolated to ensure it is not accidentally disturbed. | M 52 | Worker |
|  | Removal of Asbestos | Exposure of workers and public to Asbestos Spread of Asbestos fibres | E 88 | In most circumstances, Asbestos cement products are best left undisturbed. Removal of Asbestos-cement products does not need to be undertaken unless it is suspected that they may be releasing Asbestos fibres into the air because of poor condition. Asbestos removal is a prescribed activity. | M 52 | Worker Site Supervisor |
|  | When Breffni Ceilings & Partitions Pty Ltd contracts Asbestos removal and treatment contractor. | Using contractors who are not competent Not providing adequate information to contractors | E 88 | Breffni Ceilings & Partitions Pty Ltd will only use a certified Asbestos removalist to undertake removal of any amount of friable Asbestos or more than 10 square metres of non-friable Asbestos. The contractor’s licence will be sighted before work begins. They will liaise with the contractor during the term of the contract. Breffni Ceilings & Partitions Pty Ltd will ensure that the removalist is provided with a copy of the Asbestos register if available. Breffni Ceilings & Partitions Pty Ltd will provide work specifications which detail what is to be removed, where it is located and any hazards that may be associated with the Asbestos removal work. | M 52 | Site Supervisor |
|  | When Breffni Ceilings & Partitions Pty Ltd contracts Asbestos removal and treatment contractor | Using contractors who are not competent | E 80 | A SWMS for Asbestos removal must be prepared by the Asbestos removalist who is the holder of a certificate for performing Prescribed Activities. The Contractor’s SWMS must include: - What prescribed activity is being performed; - If the holder has an ABN, the ABN; - The certificate number of the certificate under section for performing the prescribed activity; - The specific control measures the holder proposes to use to discharge the holder’s workplace health and safety; - Obligations for the activity; - The way the holder proposes to perform the activity, including how the control measures are to be used; - How the effectiveness of the control measures will be monitored and reviewed; - The arrangements for appropriate training of workers employed, or otherwise allowed, to perform the activity; and - The arrangements for supervision of the performance of the activity by a competent person. | M 52 | Contractor |
|  | Preparing to disturb bonded Asbestos | Contact with Asbestos containing material/fibres | E 88 | Perform risk assessment – Reduce exposure to airborne asbestos products as much as possible. Assess ventilation – work in a well-ventilated area but avoid working in a windy environment. Shut down any heating or cooling systems if they are likely to spread Asbestos fibres. Place polythene sheet over any areas that could be contaminated. Secure with duct tape or similar. Ensure dust or debris is contained when sheet is removed. Wet down the work area to reduce release of dust. | M 52 | Worker Site Supervisor |
|  | Choosing tools | Creating airborne Asbestos fibres | E 88 | Use hand tools. Power tools should not be used unless they have been approved by the relevant statutory authority for asbestos work as they produce dangerous dusts. | M 52 | Worker Site Supervisor |
|  | Choosing PPE | Inhalation of Asbestos fibre, carrying Asbestos fibres out of the work area. | E 88 | Preferably wear full covering disposable overalls. Wear gloves. If they become contaminated with Asbestos waste they should be treated as such and destroyed. Use a P1 or better disposal particulate respirator in accordance with the manufacturer’s instructions. | M 52 | Worker |
|  | Drilling or cutting bonded Asbestos – walls | Creating airborne Asbestos fibres Contact with Asbestos containing materials/fibres | E 88 | Work on asbestos sheeting will be kept to as minimal as possible. If extensive work on asbestos sheeting is required the entire sheet will be removed and replaced with non-asbestos material. Cover the area to be cut of drilled and the exit point if possible with masking tape or duct tape then cover the tape with thickened substance (i.e. wallpaper paste or hair gel) to reduce the effect of dust and particles. Spray water on the drill point or saw edge. Wipe area, tools and equipment with wet rag after use. Seal the cut edges with a sealant such as PVA paint. | M 52 | Worker Contractor |
|  | Drilling bonded Asbestos – Ceilings | Creating airborne Asbestos fibres Contact with Asbestos containing material/fibres. | E 88 | When drilling or cutting into ceilings preferably work from above using appropriate fall protection. Ensure all residues are contained. Use a plastic/paper cup lined with thickened substance (i.e. wallpaper paste or hair gel) to drill through surface. Remove the drill bit from the cup before removing cup from the ceiling. Wipe area, tools and equipment with wet rag after use. | M 52 | Worker Site Supervisor |
|  | Switch boards and meter panels | Exposure of workers and public to Asbestos. Spread of Asbestos fibres Long term illness | E 80 | Wet down the work area to reduce the release of dust. If the work includes a meter/switchboard panel or is in the vicinity of live equipment ensure that supply is isolated before work commences. Ensure that all surfaces are dry before supply is restored. | M 52 | Worker |
|  | Working in ceilings containing fibro cement sheeting | Exposure of workers and public to Asbestos Spread of Asbestos fibres Long term illness. | E 88 | When working in roofs containing Asbestos products basic protection such as a face mask and full body overalls should be considered. | M 52 | Worker Site Supervisor |
|  | Cleaning | Taking Asbestos fibres outside of the workplace | E 80 | Do not use high pressure cleaning methods such as water blasting. Wet all Asbestos waste. Use wet rags to clean up any debris and place wet rags in a plastic bag and treat as Asbestos waste. If it is necessary to vacuum the area use a vacuum cleaner which is suitable for use with Asbestos material. A domestic vacuum cleaner should not be used in any circumstance. Ensure that all surfaces are dry before supply is restored. | M 52 | Worker Site Supervisor |
|  | Disposal of Asbestos waste | Exposure of workers and public to Asbestos. Spread of Asbestos fibres. Long term illness. | E 80 | Dispose of tool rags, PPE and clothing as contaminated material. All contaminated material (tools, rags, clothing and PPE) should be disposed of in a marked bag and should be double bagged. Waste to be dampened to reduce dust and bags only to be half filled. Contact should be made with the local Municipal Council to determine what facilities they have for the disposal of Asbestos waste. | M 52 | Worker Contractor Site Supervisor |
|  | Site Emergency | Poor or delayed response in the case of emergency | E 80 | Site specific emergency procedures will be followed at all times. | L 40 | Worker Site Supervisor |

| - RISK ASSESSMENT TOOL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RISK MATRIX |  |  |  |  |  |  |  |  | HIERARCHY OF CONTROLS |
| LIKELIHOOD |  |  | CONSEQUENCE |  |  |  |  |  |  |
|  |  |  | 1 |  | 2 | 3 | 4 | 5 |  |
|  |  |  | Insignificant |  | Minor | Moderate | Major | Catastrophic |  |
| A. | Almost certain |  | M 52 |  | H 64 | E 76 | E 88 | E 100 |  |
| B. | Likely |  | M 44 |  | H 56 | H 68 | E 80 | E 92 |  |
| C. | Possible |  | L 36 |  | M 48 | H 60 | E 72 | E 84 |  |
| D. | Unlikely |  | L 28 |  | L 40 | M 52 | H 64 | E 76 |  |
| E. | Rare |  | L 20 |  | L 32 | M 44 | H 56 | H 68 |  |
| SCORE INDICATOR |  |  |  |  |  |  |  |  |  |
| SCORE |  | LEVEL OF DANGER |  | REQUIRED ACTION |  |  |  |  |  |
| 72 – 100 |  | EXTREME |  | NO WORK SHOULD COMMENCE AT THIS LEVEL! This is an unacceptable level of risk. A review of controls must be performed to reduce risk as low as reasonably practicable. If risk cannot be further reduced, a Permit System must be applied. The permit must be available at all times work is being performed, signed by both the workers, a senior management representative and the Client (if applicable) to ensure that the risk is reduced as far as practicable. |  |  |  |  |  |
| 56 – 68 |  | HIGH |  | WORK CAN COMMENCE AT THIS LEVEL IF THE CONTROL MEASURES ARE APPROVED BY THE PROJECT MANAGER OR DELEGATE The Project Manager or delegate will review existing controls to ensure that the risk is reduced to as low as is reasonably practicable. |  |  |  |  |  |
| 44 – 52 |  | MODERATE |  | WORK CAN COMMENCE WITH CONTROL MEASURES IN PLACE. Management must monitor and review the controls to ensure controls are adequately used, do not present further hazards / risks and aim to reduce the risk further if practicable. |  |  |  |  |  |
| 20 - 40 |  | LOW |  | REVIEW IF CHANGES OCCUR. Record and review if any equipment, people, materials, work processes, or procedures change. |  |  |  |  |  |

| - MONITORING AND REVIEW OF CONTROL MEASURES |  |
| --- | --- |
| Pre-Start | Pre-start meetings are held to review control measures and ensure workers are aware of controls. |
| Ongoing Monitoring | Site Supervisor will continually monitor works against the controls stated in this safe work method statement. The Health and Safety Checklist is used to document formal inspections of the works and work area. |
| Annual | This safe work method statement is reviewed annually or as required. |
| NON-CONFORMANCE | Should any aspect of this SWMS not be complied with, work affected by the non-compliance must cease. The Site Supervisor will either enforce the controls as stated by the SWMS or, where appropriate, review the SWMS in consultation with workers and update with more suitable controls provided the revised controls still ensure that the hazards / risks are controlled as far as reasonably practicable. |

| - SWMS CONSULTATION AND SIGN-OFF RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I confirm, by signing below, that I have read and understand the requirements of this SWMS, I agree that the SWMS is acceptable for the proposed work, and I agree to abide by the requirements. If the work conditions change, or additional hazards are identified, these changes shall be discussed, the SWMS modified and signed again. |  |  |  |  |  |
| ISSUE | NAME (Please Print) | SIGNATURE | ISSUE | NAME (Please Print) | SIGNATURE |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |
| A |  |  |  |  |  |