# SWMS 116 – NON-STRUCTURAL DEMOLITION

| NOTE: This industry standard SWMS template has been made accessible by Breffni Ceilings & Partitions Pty Ltd, to be used as a guide only. The sub-contractor/user is entirely responsible for its contents at the point of submission. Therefore, please ensure that every section is reviewed and personalised in full. |  |  |  |  |
| --- | --- | --- | --- | --- |
| - PROJECT DETAILS |  |  |  |  |
| Project Name: |  | Project Address: |  |  |
| Client Name: |  |  |  |  |
| Description of Work (Scope): | COMMERCIAL FITOUT |  | DOORS |  |
|  | PARTITIONS |  | PLASTERBOARD |  |
|  | FLOORING |  | HOT WORKS |  |
|  | CEILINGS |  | OTHER |  |
|  | ALUMINIUM FRAMING |  |  |  |
|  | GLASS |  |  |  |
| High Risk Construction Work: | Removal of existing plasterboard and other non-supportive structures from ceilings and walls |  |  |  |
| Date Work is to Commence: |  | Duration of Work: |  |  |
| Person Responsible for Monitoring Activity: |  | Name: |  |  |
| Job Number: |  | Issue: |  | A |

| - CONSULTATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| THIS PROJECT RISK ASSESSMENT HAS BEEN DEVELOPED IN CONSULTATION WITH THE FOLLOWING WORKERS: |  |  |  |  |  |  |  |
| Issue: | Name: | Position: | Date: | Issue: | Name: | Position: | Date: |
| A |  |  |  |  |  |  |  |

| - MONITORING AND REVIEW OF CONTROL MEASURES |  |
| --- | --- |
| Pre-Start | Pre-start meetings are held to review control measures and ensure workers are aware of controls. |
| Ongoing Monitoring | Project Manager will continually monitor works against the controls stated in this safe work method statement. The Health and Safety Checklist is used to document formal inspections of the works and work area. |
| Annual | This safe work method statement is reviewed annually or as required. |
| NON-CONFORMANCE | Should any aspect of this SWMS not be complied with, work affected by the non-compliance must cease. The Project Manager will either enforce the controls as stated by the SWMS or, where appropriate, review the SWMS in consultation with workers and update with more suitable controls provided the revised controls still ensure that the hazards / risks are controlled as far as reasonably practicable. |

| - PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uniform | Footwear | High Vis’ | Safety Helmet | Protective Eyewear | Face Shield | Fall Arrest | Gloves | Hearing | Dust Mask | Respirator | Other | PPE. Notes (e.g. P2 Dust Mask, etc.) |
|  |  |  |  |  |  |  |  |  |  |  |  | Other mandatory site PPE as advised by Client or PC. |

| - PLANT AND EQUIPMENT REQUIRED FOR THIS ACTIVITY | Maintenance/Checks/Tests |
| --- | --- |
| Electric Tools | 3 monthly electrical test and tag, visual inspection before use to ensure in good condition. |
| Elevating Work Platform (EWP) | Prestart Check as per Logbook. |
| Travel Restraint Equipment | 6 and 12 monthly inspection, visual inspection before use to ensure in good condition. |
| Fall Arrest Equipment | 6 and 12 monthly inspection, visual inspection before use to ensure in good condition. |
| Ladders/trestles/planks | Visual inspection before use to ensure in good condition. |
| Glass suckers/vacuum lifters | Visual inspection before use to ensure in good condition. Test load lift close to ground prior to use. |
| Angle Grinder | 3 monthly electrical test and tag, visual inspection before use to ensure in good condition. |

| - CHEMICALS |  |  |  |
| --- | --- | --- | --- |
| Chemical Name | Hazardous? | SDS Attached | Controls |
| Plaster compounds | Yes No | Yes No | As per Safety Data Sheet |
| Silicone | Yes No | Yes No | As per Safety Data Sheet |
| Adhesives | Yes No | Yes No | As per Safety Data Sheet |
| Liquid Nail | Yes No | Yes No | As per Safety Data Sheet |

| - PERMITS, TICKETS or LICENCES REQUIRED TO PERFORM WORK |  |  |
| --- | --- | --- |
| Role / Task | Training, Permit, Ticket or Licence |  |
| Workers | Company Induction, General Construction Induction |  |
| Working from over 2m | General Work from Heights Training |  |
| Operating EWP | EWP Competency Certificate OR Crane and Hoist operation - Class: WP |  |
| 6 and 12 monthly Inspection of Travel restraint systems | Height Safety Equipment Inspector |  |
| 6 and 12 monthly Inspection of fall arrest systems | Height Safety Equipment Inspector |  |

| - LEGISLATION AUSTRALIAN STANDARDS AND CODES OF PRACTICE TO BE COMPLIED WITH |  |  |  |  |
| --- | --- | --- | --- | --- |
| QUEENSLAND |  |  | NATIONAL |  |
| Work Health and Safety Act 2011 |  |  | AS/NZS 1891.1:2007/Amdt 2:2008 Industrial Fall-Arrest Systems and Devices - Harnesses and ancillary equipment |  |
| Work Health and Safety Regulation 2011 |  |  | AS/NZS 1891.3:1997 Industrial Fall-Arrest Systems and Devices - Fall Arrest Devices |  |
| Electrical Safety Act 2002 (Qld) |  |  | AS/NZS 1891.4:2009 Industrial Fall-Arrest Systems and Devices - Selection, Use and Maintenance |  |
| Electrical Safety Regulation 2013 (Qld) |  |  | AS 2550.1-2011 Cranes, hoists and winches – Safe use – General Requirements |  |
| Electrical Safety Code of Practice 2010 – Working Near Overhead and Underground Electric Lines |  |  | AS 2550.10-2006/Amdt 1-2009 Cranes, hoists and winches - Safe use - Mobile elevating work platforms |  |
| Managing Noise and Preventing Hearing Loss at Work Code of Practice 2011 |  |  | AS 1418.10:2011/Amdt 1:2017 Cranes, hoists and winches – Mobile elevating work platforms |  |
| Hazardous Manual Tasks Code of Practice 2011 |  |  | AS/NZS 1577:2013 Scaffold decking components |  |
| Managing the Risk of Falls at Workplaces Code of Practice 2011 |  |  | AS/NZS 1576.1:2010 Scaffolding – General Requirements |  |
| How to Manage Work Health and Safety Risks Code of Practice 2011 |  |  | AS/NZS 3760 :2010/Amdt 2:2012 In-Service Safety Inspection and Testing of Electrical Equipment |  |
| Managing Risks of Hazardous Chemicals in the Workplace Code of Practice 2013 |  |  | AS/NZS 1418.10:2011 Cranes, hoists and winches – Mobile Elevating Work Platforms |  |
|  |  |  | AS/NZS 4576:1995 Guidelines for Scaffolding |  |
| Scaffolding Code of Practice 2011 |  |  | AS/NZS 1892.5: 2000 Portable ladders - Selection, safe use and care |  |
| How to Manage Work Health and Safety Risks Code of Practice 2011 |  |  | AS/NZS 1562.1-2006 Design and installation of sheet roof and wall cladding |  |
| Managing Risks of Plant in the Workplace Code of Practice 2013 |  |  |  |  |
| WHS Consultation, Cooperation and Coordination Code of Practice 2011 |  |  |  |  |
| How to Manage and Control Asbestos in the Workplace Code of Practice 2011 |  |  |  |  |
| How to Safely Remove Asbestos Code of Practice 2011 |  |  |  |  |
| - APPROVAL |  |  |  |  |
| I hereby accept the controls as detailed in the below risk assessments and confirm that controls will be implemented and complied with. |  |  |  |  |
|  | APPROVED BY | REVIEWED PRIOR TO WORK COMMENCING (Client/PC Representative) |  | REVIEWED ON SITE PRIOR TO COMMENCING WORK (Project Manager) |
| NAME |  |  |  |  |
| SIGNATURE |  |  |  |  |
| DATE |  |  |  |  |

## Site Set Up / General

| PROJECT RISKS AND CONTROLS – Tick  or cross  the far left column to indicate whether the step is relevant to this project |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  or  | TASK/ACTIVITY STEPS | HAZARD/RISK | RISK SCORE BEFORE | CONTROL MEASURES | RISK SCORE AFTER |
|  | Drive to Site | Unlicensed operator. | E 72 | Project Manager to ensure only licensed drivers operate vehicles. Workers will be trained in (at Toolbox consultations) and perform work according to Work Instruction – Motor Vehicles. | L 20 |
|  | Induct relevant people | Workers unaware of important safety information. | H 60 | Workers will have undergone general induction and will undergo a site induction. Toolbox consultation meetings will be held to induct workers into relevant SWMS. | L 40 |
|  | - Assessment of the site | Not being aware of hazards present. | H 60 | Project Manager will visually inspect site surroundings, property access, weather conditions, traffic or other hazards prior to workers entering the worksite. Where particular issues are identified this will be communicated at Toolbox Consultations. Workers should review the outcome of the assessment to ensure they are aware of any site hazards or conditions relevant to the work being performed. | L 28 |
|  | Entering the work site | Not being aware of hazards present. | H 68 | Workers will observe all signage requirements including all mandatory site personal protective equipment. Workers will participate in Breffni Ceilings & Partitions and Principal Contractor Prestart Meetings and toolbox consultations where applicable. Workers will continually assess their work environment for hazards. If hazards beyond their control are identified these will be reported to the Project Manager. Project Manager to investigate and control identified hazards. If beyond his/her control, Project Manager to complete a Safety Observation / Hazard Report and discuss with Client/Principal Contractor or Director. | L 40 |
|  |  | Persons not wearing appropriate PPE. | H 60 | Workers must wear mandatory site Personal Protective Equipment (PPE) and as required for the tasks they are performing. Project Manager will ensure that PPE requirements are complied with. | L 40 |
|  |  | Falls to same level. (Slips, trips, falls over objects, etc.) | H 60 | Workers will maintain a clear, clean working area. Project Manager will monitor work area. | L 40 |
|  |  | Works affecting other trades. | E 72 | Project Manager will advise any persons who may be affected by work for instance by noise, dust etc. throughout the project. | L 40 |
|  | - Identifying Asbestos | Inhalation of Asbestos fibres Exposure of workers and public to Asbestos | E 88 | Many homes built before 1980 contain asbestos in old floor tiles, ceiling tiles, roof shingles and flashing, siding, insulation (around boilers, ducts, pipes, sheeting, fireplaces), pipe cement, and joint compound used on seams between pieces of sheetrock. Some newer houses may also contain asbestos. If any work is to be conducted around Asbestos, workers should cease work immediately and contact Breffni Ceilings & Partitions’ management without delay. Workers will be trained in (at Toolbox consultations) and perform work according to Work Instruction – Asbestos Awareness Guide. | M 52 |
|  | Delivery of materials on site | Blocking safe access/egress. Damage /loss of materials. | H 60 | Project Manager will consult with the Client or Principal Contractor to identify a suitable lay-down area. Workers to use designated unloading platforms where available. Project Manager to monitor work processes. | L 32 |
|  |  | Manual handling | H 68 | Workers will be trained in (at Toolbox consultations) and perform work according to with Work Instruction – Manual Tasks. All workers to be made aware of safe manual handling techniques. Mechanical lifting equipment will be used if possible. An adequate number of workers will be available to assist with heavy loads. | L 32 |
|  |  | Impact with vehicles Impact with other workers | E 84 | Workers are not to walk behind a truck unless it is turned off. Project Manager to assess if barricading is required for public/other trades’ safety. Refer Work Instruction – Signs and Barricades. | M 44 |
|  | Workers to take materials to a safe storage area | Manual handling of materials. | H 68 | Workers are trained in and will utilise safe manual handling techniques. Workers will limit items carried, utilise mechanical aids, i.e. trolleys or team lift heavy items. Workers are required to report if they are not fully fit for work, due to any form of injury, to their Project Manager. Project Manager will ensure compliance to the Hazardous Manual Tasks Code of Practice | L 32 |
|  |  | Trips | H 60 | Workers to check path of travel prior for hazards. Clear or find alternative path if it is blocked, congested or not well lit. Workers to ensure walkways are kept clear. Project Manager to monitor work processes. | L 32 |
|  |  | Cuts, irritation. | H 60 | Workers to wear gloves, eyewear, long sleeves and long pants as appropriate. Workers will be trained in (at Toolbox consultations) and perform work according to Work Instruction – Personal Protective Equipment. Project Manager will monitor work to ensure compliance. | L 32 |
|  | Set out area | Public affected by works. | E 72 | Workers will take special precautions to stop or minimise contact where there is a likelihood that the general public may come into contact with works. Where this contact would be over several hours, then barricading and signage will be implemented. For short-term contacts, a dedicated observer may be utilised. Project Manager will ensure that signage and barricading requirements are complied with. Workers will be trained in (at Toolbox consultations) and perform work according to Work Instruction – Working Around the Public and Work Instruction – Signs and Barricades. | L 40 |
|  | Using hand tools or power tools | Musculoskeletal injury Noise Vibration Cuts, abrasions Contact with moving parts | E 72 | Workers will be trained in and comply with Work Instruction – Hand Tools and Work Instruction – Power Tools. Inspect tools before use. Do not use faulty tools. Ensure the tool is appropriate to the job. Only persons who are trained in the safe use of power tools are permitted to use them. Ensure all guards are in place and operate correctly. Ensure that PPE is being worn i.e. eye protection, hearing protection. | L 40 |
|  | Using electrical equipment | Electrical shock | E 88 | Before using any electrical equipment ensure it has been inspected and tested by a licensed electrician and has a current inspection tag affixed. Electrical equipment will also be connected to a safety switch. Electrical leads will be elevated to avoid crossing paths or access ways. Leads should have no visible damage to the protective cover. Leads will be maintained to avoid kinking. Workers will be trained in (at Toolbox consultations) and perform work according to Work Instruction – Electrical Equipment. | L 40 |
|  | Using angle grinders | Incorrect or faulty tool Electrical shock Kickback Untrained workers Eye injury from moving parts Noise | E 88 | Assess whether an angle grinder is the best tool to perform the task or whether the use of another tool could be more effective. Inspect angle grinder before use. Do not use faulty angle grinder. Angle grinders should be fitted with an automatic cut-off or “dead-man” switch as part of the handgrip to isolate power as soon as finger pressure is released. Ensure the correct type of cutting disk for the task is selected. The wrong disc could shatter the work material causing kickback. Only persons who are trained in the safe use of angle grinders are permitted to use them. Ensure all guards are in place and operate correctly. Ensure that PPE is being worn i.e. eye protection, hearing protection. | L 40 |
|  |  | Incorrect use of angle grinder | E 88 | Ensure the angle grinder is only used for purposes for which it was designed. Always follow the work instruction and operator’s instructions. Ensure a handle and guard is always fitted to the angle grinder. Do not use the angle grinder as a cutting tool. | L 40 |
|  |  | Fire or explosion | H 60 | Do not use angle grinder where there is risk of causing fire or explosion i.e. in hazardous areas. Ensure fire extinguisher is located in close proximity to the works being undertaken. | L 40 |
|  |  | Musculoskeletal / manual handling injuries | E 88 | Where possible keep work at waist height during grinding and adopt a comfortable stance with feet apart for balance. Workers to ensure they have a clear view of the grinding task at all times. Workers to stop at regular intervals for short breaks to rest hands and arms in between grinding. | L 40 |
|  | Lifting heavy equipment / materials | Musculoskeletal injury | H 68 | Workers will be trained in (at Toolbox consultations) and perform work according to with Work Instruction – Manual Tasks. All workers to be made aware of safe manual handling techniques. Mechanical lifting equipment will be used if possible. An adequate number of workers will be available to assist with heavy loads. | L 40 |
|  | - Working from heights using ladders and trestles | Electric shock Fall from heights Musculoskeletal injury | E 88 | Inspect ladder for defects, do not use damaged equipment. Only use ladders that are owned by Breffni. Do not loan equipment to others. Make sure that ladders are: - In good condition; - An appropriate height to comfortably perform the work required; - Have a rated capacity of at least 120kg; - Identified by the manufacturer as “for industrial use”; - Used only for the purpose for which it was designed; - Non-conductive (i.e. fibreglass) if there is a risk of electrocution; and - No longer than 6.1 metres for single ladders and 7.5 metres for extension ladders (9.2 m for an extension ladder used to do electrical work). - Workers will be trained in (at Toolbox consultations) and perform work according to Work Instruction - Ladders. | M 52 |
|  |  | Slips, trips and falls | H 60 | Make sure area is clear of obstacles. | L 40 |
|  |  | Injuries from falling objects | H 60 | Erect appropriate barricading and signage to exclude others from entering the area below elevated work. Tethers must be used to tie-off tools, equipment and materials. | L 40 |
|  |  | Falls from height | E 88 | Only light fixing work to be completed using ladders and trestles. Trestles to be supported by ladders or stands only from ground level. Do not set up on uneven or sloping ground. Workers will be trained in (at Toolbox consultations) and perform work according to Work Instruction – Using Trestles and Work Instruction – Ladders. | L 40 |
|  |  | Ladder sliding out Ladder tilting User loss of balance | E 80 | Position ladders: - On a stable and level surface free from obstacles; - So that the rungs are approximately level; - So that the angle between the ladder and the horizontal is at least 70º but not more than 80º when in use; - So that the end extends 1 metre above a surface if it is being used as a means of temporary access. Secure the ladder by tying off to a firm upright surface or secured by the use of bracing. Where a tie off point is not available, use a second person to support the ladder whilst in use. Make sure only one person is on the ladder at one time. Make sure 3 limbs are holding, wrapped around or standing on the ladder at all times, unless otherwise secured by a means of fall protection. Do not stand higher than the third rung from the top of the ladder. Make sure that your body remains within the stiles of the ladder. Avoid stretching and extending. Face the ladder when climbing up or down. |  |
|  | Using Hazardous Chemicals | Fire Leaks or spills Asphyxiation Burns Long term illness | E 80 | Ensure a Safety Data Sheet (SDS) is available for each substance used. Ensure the SDS is not more than five years old and meets GHS requirements. Dispose of chemical no longer in use according to the manufacturer’s recommendations or the SDS. Read and follow prescribed safety procedures as stated on SDS’s. Wear recommended PPE at all times when working with hazardous chemical. Report symptoms resulting from the use of hazardous chemical to the Project Manager immediately. Workers will be trained in (at Toolbox consultations) and perform work according to Work Instruction – Hazardous Chemicals. | L 40 |
|  |  | Poor understanding of chemical hazards Failure to implement appropriate control measures | E 80 | Ensure a risk assessment for the hazardous chemical is undertaken, a safe operating procedure is developed that outlines how the substance is to be used safely and the procedure for the chemicals’ safe use is communicated, understood and followed by the employees using these hazardous chemicals. The procedure will be monitored and periodically reviewed. | L 40 |
|  |  | Spills | H 68 | Make sure equipment is available to clean spills up i.e. spill kit. Clean up spills as soon as possible. Isolate area from public or other persons. Dispose of as directed by the SDS. | L 40 |
|  | Storage of chemicals | Unidentified chemicals | E 80 | Ensure every chemical is clearly labelled with details of the nature and hazard of the chemical. Do not store chemical in beverage containers. | L 40 |
|  |  | Reaction of incompatible goods Fire, leaks or spills, burns | E 90 | Do not use or store incompatible goods together. Make sure flammable, explosive or toxic chemical are stored away from possible sources of electrical spark, heat or flame. | M 52 |

## Non-Structural Demolition

| PROJECT RISKS AND CONTROLS – Tick  or cross  the far left column to indicate whether the step is relevant to this project |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  or  | TASK/ACTIVITY STEPS | HAZARD/RISK | RISK SCORE BEFORE | CONTROL MEASURES | RISK SCORE AFTER |  |
|  | Remove partitions / ceilings | Manual Handling | H 60 | Workers will use correct lifting or manual handling techniques, team lifting or mechanical lifting aids. Project Manager will monitor compliance to safe procedures. |  | L 40 |
|  |  | Electric shock | E 88 | Power to be isolated to area of demolition by cert electrician. |  | M 52 |
|  |  | Objects hitting other workers (Falling tools, material, etc.) | E 72 | Workers will advise the Project Manager where work is to be performed above other workers. This will also be discussed during pre-work meetings with the contractor. Project Manager will schedule work so work is not performed above other workers. |  | L 40 |
|  |  | Damage to other services | E 72 | All services (water, phone communications) are to be disconnected by competent persons before demolition works commence. |  | L 40 |
|  |  | Personal damage (Hand, eye and lung damage, etc.) | E 72 | Wear appropriate personal protective equipment. (Gloves, goggles, respiratory, etc.) |  | L 40 |

## Removal of Existing Floor Coverings

| PROJECT RISKS AND CONTROLS – Tick  or cross  the far left column to indicate whether the step is relevant to this project |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  or  | TASK/ACTIVITY STEPS | HAZARD/RISK | RISK SCORE BEFORE | CONTROL MEASURES | RISK SCORE AFTER |
|  | Removal of furniture | Manual handling | H 68 | Workers are to be trained in and utilise safe manual handling techniques - mechanical lifting equipment will be used if possible. Workers will be trained in (at Toolbox consultations) and perform work in accordance with Work Instruction Manual Tasks. Project Manager will monitor to ensure compliance. | L 40 |
|  | Removal of existing vinyl and ceramic tiles using jack hammer | Vibration – impaired blood circulation | H 68 | Inspect the tool prior to use. Limit time spent using the jack hammer. When extended use is called for workers will rotate and take adequate breaks between uses. Take rest breaks as needed. | L 40 |
|  |  | Inhalation of dust | H 56 | Wear safety glasses. Wear dust mask if the operation produces dust and ventilation is inadequate. | L 40 |
|  |  | Manual handling | E 72 | Warm up to prevent back injury. Limit time spent using the jack hammer. When extended use is called for workers will rotate and take adequate breaks between uses. | L 40 |
|  |  | Works affecting other trades | H 68 | Advise any persons who may be affected by your work for instance by noise, dust etc. | L 40 |
|  |  | Hearing damage | H 60 | Wear ear muffs. Warn others in the vicinity if they may need to wear hearing protection. | L 40 |
|  | Removal of existing vinyl using Floor Stripper | Vibration – impaired blood circulation | H 68 | Limit time spent using the floor stripper. When extended use is called for workers will rotate and take adequate breaks between uses. Take rest breaks as needed. Always disconnect the electric power before altering blade bevel. Make sure all bolts and fasteners are tightened. Due to the normal vibration of floor strippers, fasteners can come loose. | L 40 |
|  |  | Manual handling | E 72 | Prevent back injuries by using your leg muscles to lift the machine into operating position. Allow the tool to do the work by using a grip light enough to maintain control. Be sure to keep good firm footing and maintain good balance. Do not overreach when operating the floor stripper. Use correct lifting or manual handling techniques. Seek assistance with any manual handling task that you feel you cannot do safely. If you are not fully fit for work, due to any form of injury, report this to your Project Manager. Project Manager will ensure compliance to the Hazardous Manual Tasks Code of Practice. | L 40 |
|  |  | Works affecting other trades/ Public | H 68 | Advise any persons who may be affected by your work for instance by noise, dust etc. Barricade work area to exclude unauthorised persons. | L 40 |
|  |  | Using damaged tools | H 60 | Inspect drill before use, i.e. ensure cutting tools they are sharp and clean, cord is free from damage, and handles are dry, clean and free from oil and grease. If tool is damaged – do not use. Tag ‘Out of Service’, remove from work area and have it repaired by an authorised service facility. | L 40 |
|  |  | Electric shock | H 60 | Ensure floor stripper has a current test tag affixed. All electrical tools must be connected to a safety switch. If stopping work for a short period of time or for the day, unplug the electricity. | L 32 |
|  | Removal of existing vinyl using hand scrapers | Using hand tools. | M 48 | Workers will inspect tools for damage before use. Workers will scrape away from themselves where possible. Project Manager will ensure equipment provided for use is appropriate and in good condition. | L 28 |
|  | Using Floor Grinder to remove adhesive | Vibration – impaired blood circulation | H 68 | Inspect the tool prior to use. Limit time spent using the jack hammer. When extended use is called for workers will rotate and take adequate breaks between uses. Take rest breaks as needed. | L 40 |
|  |  | Inhalation of dust | H 56 | Wear safety glasses. Wear dust mask if the operation produces dust and ventilation is inadequate. | L 40 |
|  |  | Manual handling | E 72 | Warm up to prevent back injury. Limit time spent using the jack hammer. When extended use is called for workers will rotate and take adequate breaks between uses. | L 40 |
|  |  | Works affecting other trades | H 68 | Advise any persons who may be affected by your work for instance by noise, dust etc. | L 40 |
|  |  | Hearing damage | H 60 | Wear ear muffs. Warn others in the vicinity if they may need to wear hearing protection. | L 40 |
|  | Using solvent to remove left over adhesive | Hazardous Chemicals | M 48 | Workers will assess the ventilation requirements and use in well-ventilated area or provide mechanical ventilation and wear appropriate PPE including vapour respirator, safety goggles and gloves. Project Manager to ensure the Safety Data Sheet (SDS) for each hazardous substance used, review SDS and discuss at Toolbox Consultation. Workers will be trained in (at Toolbox Consultations),and perform work in accordance with Work Instruction Personal Protective Equipment be trained in, sign off on and perform work according to Work Instruction Using Chemicals. Project Manager to monitor to ensure compliance. | L 36 |
|  | Removal of carpet, underlay and smooth edge (tackstrips) | Soiled carpet | M 44 | Workers to assess condition of carpet, ensure carpet has been vacuumed to minimise dust and wear appropriate PPE, e.g. coveralls, dust masks and gloves. Project Manager to monitor to ensure compliance. | L 28 |
|  |  | Hand tools e.g. knives, prybars etc. | H 60 | Worker to always cut with the blade of the knife angled away from them. Workers will inspect tools for damage before use. Project Manager will ensure equipment provided for use is appropriate and in good condition. | L 32 |
|  | Removal of direct stick carpet and underlay | Soiled carpet | M 44 | Workers to assess condition of carpet, ensure carpet has been vacuumed to minimise dust and wear appropriate PPE, e.g. coveralls, dust masks and gloves. Project Manager to monitor to ensure compliance. | L 28 |
|  |  | Hand tools e.g. knives, etc. | H 60 | Worker to always cut with the blade of the knife angled away from them. Workers will inspect tools for damage before use. Project Manager will ensure equipment provided for use is appropriate and in good condition. | L 32 |
|  |  | Using solvent to remove left over adhesive | M 48 | Ensure a current Safety Data Sheet (SDS) is available for Concrete. Read and follow prescribed safety procedures as stated on SDS’s. Wear recommended PPE at all times when working with substances. | L 36 |

## Safety Agreement – Page 1

I have been consulted with in the creation of the above risk assessment, understand and agree to comply with the WHS Management Plan, Safe Work Method Statements, Work Instructions, site rules, and other safety strategies in place on this site. I will discuss any site-specific safety concerns with the job Project Manager.

| NAME | POSITION | INDUCTION DATE | SIGNATURE |
| --- | --- | --- | --- |

## Safety Agreement – Page 2

I have been consulted with in the creation of the above risk assessment, understand and agree to comply with the WHS Management Plan, Safe Work Method Statements, Work Instructions, site rules, and other safety strategies in place on this site. I will discuss any site-specific safety concerns with the job Project Manager.

| NAME | POSITION | INDUCTION DATE | SIGNATURE |
| --- | --- | --- | --- |